CascadableNbitComparator

Objectif

Ce composant est un comparateur de magnitude cascadable sur N bits. Il permet de comparer deux nombres en les découpant en tranches (slices) qui se chaînent, un peu comme on pose une comparaison à la main en commençant par les bits de poids faible.

🔌 Interface

Entrées

- a, b : Les deux nombres à comparer sur N bits (cette tranche)

- eq_in, gt_in, lt_in : Signaux provenant de la tranche de poids inférieur, indiquant le résultat de la comparaison des bits plus faibles

Sorties

- eq_out, gt_out, lt_out : Résultat combiné de cette tranche ET des tranches inférieures

Logique interne

1. Comparaison locale (eq_self, gt_self, lt_self)

Le composant compare d'abord les N bits de cette tranche uniquement :

vhdl

eq_self <= '1' quand a_unsigned = b_unsigned

gt_self <= '1' quand a_unsigned > b_unsigned

lt_self <= '1' quand a_unsigned < b_unsigned

2. Logique de cascade (le cœur du composant)

C'est là que la magie opère :

vhdl

eq_out <= eq_in AND eq_self

On est globalement égal seulement si cette tranche est égale ET toutes les tranches inférieures étaient égales.

vhdl

gt_out <= gt_self OR (eq_self AND gt_in)

On est globalement plus grand si :

- Soit cette tranche est strictement plus grande (prioritaire)

- Soit cette tranche est égale ET les bits inférieurs étaient plus grands

vhdl

lt_out <= lt_self OR (eq_self AND lt_in)

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

USE ieee.numeric_std.ALL;

ENTITY CascadableNbitComparator IS

GENERIC (

N : INTEGER := 8 -- width of this slice

);

PORT (

a, b : IN STD_LOGIC_VECTOR(N - 1 DOWNTO 0);

eq_in : IN STD_LOGIC; -- from lower bits

gt_in : IN STD_LOGIC;

lt_in : IN STD_LOGIC;

eq_out : OUT STD_LOGIC; -- to higher bits

gt_out : OUT STD_LOGIC;

lt_out : OUT STD_LOGIC

);

END CascadableNbitComparator;

ARCHITECTURE Behavioral OF CascadableNbitComparator IS

SIGNAL a_unsigned : unsigned(N - 1 DOWNTO 0);

SIGNAL b_unsigned : unsigned(N - 1 DOWNTO 0);

SIGNAL eq_self : STD_LOGIC;

SIGNAL gt_self : STD_LOGIC;

SIGNAL lt_self : STD_LOGIC;

BEGIN

a_unsigned <= unsigned(a);

b_unsigned <= unsigned(b);

-- Compare within this slice

eq_self <= '1' WHEN a_unsigned = b_unsigned ELSE

'0';

gt_self <= '1' WHEN a_unsigned > b_unsigned ELSE

'0';

lt_self <= '1' WHEN a_unsigned < b_unsigned ELSE

'0';

-- Cascade logic

eq_out <= eq_in AND eq_self; -- only equal if self-equal AND lower bits equal

gt_out <= gt_self OR (eq_self AND gt_in); -- greater if self-greater, or self-equal and lower greater

lt_out <= lt_self OR (eq_self AND lt_in); -- same for less

END Behavioral;

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

USE ieee.numeric_std.ALL;

ENTITY CascadableNbitComparator_tb IS

GENERIC (

N : INTEGER := 8

);

END CascadableNbitComparator_tb;

ARCHITECTURE Behavioral OF CascadableNbitComparator_tb IS

-- Component declaration

COMPONENT CascadableNbitComparator IS

GENERIC (

N : INTEGER := 8

);

PORT (

a, b   : IN STD_LOGIC_VECTOR(N - 1 DOWNTO 0);

eq_in  : IN STD_LOGIC;

gt_in  : IN STD_LOGIC;

lt_in  : IN STD_LOGIC;

eq_out : OUT STD_LOGIC;

gt_out : OUT STD_LOGIC;

lt_out : OUT STD_LOGIC

);

END COMPONENT;

-- Test signals

SIGNAL a_tb, b_tb       : STD_LOGIC_VECTOR(N - 1 DOWNTO 0);

SIGNAL eq_in_tb         : STD_LOGIC;

SIGNAL gt_in_tb         : STD_LOGIC;

SIGNAL lt_in_tb         : STD_LOGIC;

SIGNAL eq_out_tb        : STD_LOGIC;

SIGNAL gt_out_tb        : STD_LOGIC;

SIGNAL lt_out_tb        : STD_LOGIC;

BEGIN

-- Unit Under Test instantiation

UUT : CascadableNbitComparator

GENERIC MAP(

N => N

)

PORT MAP(

a       => a_tb,

b       => b_tb,

eq_in   => eq_in_tb,

gt_in   => gt_in_tb,

lt_in   => lt_in_tb,

eq_out  => eq_out_tb,

gt_out  => gt_out_tb,

lt_out  => lt_out_tb

);

-- Stimulus process

stim_proc : PROCESS

BEGIN

-- Initialize inputs from lower bits (LSB slice: eq_in=1, others=0)

eq_in_tb <= '1';

gt_in_tb <= '0';

lt_in_tb <= '0';

-- Test 1: a = b (equal values)

a_tb <= std_logic_vector(to_unsigned(85, N));

b_tb <= std_logic_vector(to_unsigned(85, N));

WAIT FOR 10 ns;

ASSERT (eq_out_tb = '1' AND gt_out_tb = '0' AND lt_out_tb = '0')

REPORT "Test 1 (a=b) failed"

SEVERITY ERROR;

-- Test 2: a > b

a_tb <= std_logic_vector(to_unsigned(200, N));

b_tb <= std_logic_vector(to_unsigned(150, N));

WAIT FOR 10 ns;

ASSERT (eq_out_tb = '0' AND gt_out_tb = '1' AND lt_out_tb = '0')

REPORT "Test 2 (a>b) failed"

SEVERITY ERROR;

-- Test 3: a < b

a_tb <= std_logic_vector(to_unsigned(50, N));

b_tb <= std_logic_vector(to_unsigned(175, N));

WAIT FOR 10 ns;

ASSERT (eq_out_tb = '0' AND gt_out_tb = '0' AND lt_out_tb = '1')

REPORT "Test 3 (a<b) failed"

SEVERITY ERROR;

-- Test 4: a = b but lower bits say greater (cascade test)

a_tb <= std_logic_vector(to_unsigned(100, N));

b_tb <= std_logic_vector(to_unsigned(100, N));

eq_in_tb <= '1';  -- lower bits equal

gt_in_tb <= '1';  -- but lower slice had a>b

lt_in_tb <= '0';

WAIT FOR 10 ns;

ASSERT (eq_out_tb = '1' AND gt_out_tb = '1' AND lt_out_tb = '0')

REPORT "Test 4 (a=b, lower gt) failed"

SEVERITY ERROR;

-- Test 5: a = b but lower bits say less (cascade test)

a_tb <= std_logic_vector(to_unsigned(100, N));

b_tb <= std_logic_vector(to_unsigned(100, N));

eq_in_tb <= '1';

gt_in_tb <= '0';

lt_in_tb <= '1';  -- lower slice had a<b

WAIT FOR 10 ns;

ASSERT (eq_out_tb = '1' AND gt_out_tb = '0' AND lt_out_tb = '1')

REPORT "Test 5 (a=b, lower lt) failed"

SEVERITY ERROR;

-- Test 6: a > b regardless of lower bits

a_tb <= std_logic_vector(to_unsigned(255, N));

b_tb <= std_logic_vector(to_unsigned(128, N));

eq_in_tb <= '1';

gt_in_tb <= '1';

lt_in_tb <= '1';

WAIT FOR 10 ns;

ASSERT (eq_out_tb = '0' AND gt_out_tb = '1' AND lt_out_tb = '0')

REPORT "Test 6 (a>b, irrelevant lower) failed"

SEVERITY ERROR;

-- Test 7: Boundary values (max vs min for 8-bit)

a_tb <= (OTHERS => '1');  -- 255

b_tb <= (OTHERS => '0');  -- 0

eq_in_tb <= '1';

gt_in_tb <= '0';

lt_in_tb <= '0';

WAIT FOR 10 ns;

ASSERT (eq_out_tb = '0' AND gt_out_tb = '1' AND lt_out_tb = '0')

REPORT "Test 7 (boundary max>min) failed"

SEVERITY ERROR;

-- Test 8: Test with all zeros and eq_in=0 (should propagate)

a_tb <= (OTHERS => '0');

b_tb <= (OTHERS => '0');

eq_in_tb <= '0';

gt_in_tb <= '0';

lt_in_tb <= '0';

WAIT FOR 10 ns;

ASSERT (eq_out_tb = '0' AND gt_out_tb = '0' AND lt_out_tb = '0')

REPORT "Test 8 (all zeros, eq_in=0) failed"

SEVERITY ERROR;

-- End of simulation

REPORT "All tests completed successfully"

SEVERITY NOTE;

WAIT;

END PROCESS;

END Behavioral;